| Figure 1. The camera shoe Camera Shoe Shoe Guide Figure 2. Lowering the camera onto the PowerBase, guiding the camera shoe into the space between the shoe guide and the lock lever. Figure 3. Closing the lock lever |  | PowerBase-IR® # QuickStart Guide Thank you for purchasing a PowerBase-IR. The intent of this document is to get you up and running as quickly as possible. For a fuller explanation of the features and controls please refer to our web site. 1 What’s in the box? The package contains the following items: 1 321cam PowerBase-IR motorized base 1 Universal Remote Control 2 AAA batteries 1 Warranty card 1 External Power Supply 1 Camera shoe A camera is not included with this package. 2 What Does It Do? PowerBase-IR is a remote-controlled pan/tilt unit for any camera weighing between ¾ lb. and 3 lbs. with a standard ¼” #20 tripod mount. The PowerBase-IR is a motorized base that pans and tilts the camera. The included TV style remote control is used to pan and tilt the unit using the number buttons. #### 3 Before You Begin Insert 2 fresh AAA batteries into the included remote control unit. Verify that the top right LED lights when number pad buttons are pressed. Unwrap the included external power supply. Verify that its input voltage range includes the voltage of your electrical wall outlets. If it does, plug the external power supply unit into an electrical outlet. It is possible for the voltage to match but the plug not to fit. In this case, a plug adaptor is required, but a transformer is not required. Plug the small connector on the end of the power supply cable into the back of the PowerBase-IR. Verify that the lights on the PowerBase-IR illuminate briefly when power is applied. |
| --- | --- | --- |

| 4 Mounting your camera to the PowerBase - Screw the camera shoe, shown in Figure 1, into the ¼” screw thread on the bottom of your camera. - Mount the PowerBase-IR onto your camera tripod, treating the PowerBase-IR as if it were a camera and following the directions that came with the tripod. Do not continue without mounting your PowerBase-IR on a tripod. The weight of a camcorder or digital still camera may cause the PowerBase-IR to tip otherwise. - Move PowerBase’s lock lever counter-clockwise to its open position. - Lower your camera onto the PowerBase, guiding the camera shoe into the space between the shoe guide and the lock lever as shown in Figure 2. - Move the lock lever clock-wise all the way to its closed position, as shown in Figure 3. You should feel the camera click into place. - Make sure that your camera feels firmly locked into place. If it does not, repeat steps c-f. Do not attempt to use your PowerBase-IR before you are sure that your camera has locked into place securely. |  | #### 5 Testing default operation When the PowerBase-IR has power and the remote control unit has fresh batteries, pressing a number button (1 to 9) on the remote should cause the PowerBase-IR to move in the direction suggested by the number button’s position relative to the center 5 button. If there is no movement or the lights on the PowerBase-IR do not flash when you press buttons 1 to 9, the remote may need to be programmed. To program the remote for default operation, press and hold the SET and TV buttons simultaneously until the LED remains on. Then press 0/10 three times. The LED should turn off, indicating that the remote has been programmed. Pressing a number button (1 to 9) should now cause the PowerBase-IR to move. Pressing the 5 button should cause the PowerBase-IR to move to the Center position. #### 6 Using your own Universal Remote Control #### The remote control unit supplied with this package has limited range. If you have your own Universal Remote Control, you may prefer to use that to control your PowerBase-IR. The following steps are OPTIONAL. #### The PowerBase-IR will accept standard SONY® TV numeric commands. Consult your Universal Remote Control’s instructions if you do not know how to program it. Your Universal Remote Control’s instructions should also indicate the code to use for SONY TVs. The code is usually 000, 001, or 002. #### 7 Reprogramming your PowerBase-IR for different buttons CAUTION: The following steps are OPTIONAL and for advanced users only. You may wish to make your PowerBase-IR respond to different buttons on your remote control unit. Start by making sure your remote control is sending SONY TV commands. Enter programming mode on the PowerBase-IR by pressing and holding the Pupil Button (in the center of the arrows) for 2 seconds. You may need to use the back of a pen or pencil. Do not scratch or mark the surface of the button, as it is also the IR sensor. The UP arrow will flash to indicate programming mode. Press the remote control button you would like to control the UP direction. If the command is accepted, the LEFT and UP arrows will flash. Press the button on your remote you would like to control the diagonal UP, LEFT direction. Continue in this manner until all 4 direction arrows flash. Finally, press the button you would like to use for the Center position. After this command is accepted, the PowerBase-IR will exit programming mode. If PowerBase-IR power is lost during programming, the PowerBase-IR will revert to original factory set default programming. |
| --- | --- | --- |